Main steps to perform task 2 in ArcGIS Pro:

- Load the junctions, pipes, and meters shapefiles into GIS.

- Export the shapefiles to be feature class types, which will be used for the rest of the tutorial.

- Repeat each of the following steps for both the pipes and junctions separately.

  - To the meters feature class, add a new field called ConnectionNum and use calculate field to populate the ConnectionNum field with the OBJECTID values.

  - Use the Near tool to calculate the nearest distance to the junctions or pipes.

  - Select the input to be the meters feature class, specify the near features to be the junctions or pipes feature class, and make sure that the location box is checked.

  - Use the Make XY Event Layer tool to create points at the nearest distance calculated in the step before.

  - Using the meters as the input XY Table, select the X Field to be NEAR_X and the Y Field to be NEAR_Y.

  - Use the Append tool to associate the nearest distance points to the meters data, using the output from the previous step as the input, meters as the target dataset, and the field matching type as skip and warn if schema does not match.

  - Use the points to line tool to create lines between the meters and the closest junctions or pipes. Select the meters as the input features and the ConnectionNum field as the line field. The final output will be the meters connected to the nearest pipe or junction through lines.

References

How To: Create Connection Lines between Points and a Line in ArcGIS Pro (esri.com)

https://support.esri.com/en-us/knowledge-base/how-to-create-connection-lines-between-points-and-a-lin-000030465